- INTERNATIONAL AMATEUR RADIO UNION  Region 1

- RULES FOR CHAMPIONSHIPS IN AMATEUR RADIO DIRECTION FINDING

# Appendix 1:	Technical Specifications for Amateur Radio Direction Finding Equipment

## 1.	Receivers

- 1.1	Receivers and antennas of any type may be used by the competitors.

- 1.2	Any receiver producing audible interference in the 3,5 MHz or 144 MHz band at a distance of 10 meters or more shall not be used in the competitions.

- 1.3	The International Jury may require tests on any competitor’s receiver prior to its use in the competitions.

## 2.	Transmitters

- 2.1	It is the responsibility of the organising society to ensure that all transmitters in use comply with the relevant radio regulations in force in the country of operation with regard to their electric parameters, identification and to their being operated only by duly authorised operators.

- 2.2	All transmitters within one competition shall have the same parameters and the same antenna installation.

- 2.3	The frequency of the beacon transmitter shall be significantly different from the frequency of the other five transmitters.

- 2.4	When not scheduled to be transmitting, neither transmitter nor its antenna may radiate any RF energy.

- 2.5	Antennas shall provide omnidirectional horizontal radiation patterns.

- 2.6	Specifications for the 3.5 MHz transmitters:

- Carrier frequency *	3510 ... 3600 KHz

- Frequency stability	better than 50 ppm

- Undesired products level	conforming to the national regulations

- Output RF power 	1 - 5 W

- Mode	A1A (keyed unmodulated carrier)

- Keying speed	8 ... 12 WPM

- Antenna	vertical

- 2.7	Specifications for the 144 MHz transmitters:

- Carrier frequency *	144.5 ... 144.80 MHz

- Frequency stability	better than 50 ppm

- Undesired products level	conforming to the national regulations

- Effective Radiated Power (ERP) 	0,25 - 1 W

- Mode	A2A (keyed carrier modulated by AF tone or continuous carrier 		modulated by keyed AF tone)

- Modulation depth	75 +- 5%

- Keying speed	8 ... 12 WPM

- Antenna polarisation	horizontal

- Antenna height	2 ... 3 meters above  ground level

- Note: ( * ) Carrier frequency ranges indicated in these Rules are those normally used by IARU Region I societies. Organizers in other Regions may employ different ranges to meet their band plan regulations. According to Part A 7.4 of these Rules transmitter frequencies are to be stated in bulletin 2, which is to be dispatched 5 months before the event.

## 3.	Time-keeping system

- 3.1	The official competition time shall correspond to the official local time.

- 3.2	Two independent time-keeping systems, a primary and a secondary, shall be used.

- 3.3	The maximum deviation of the clocks at the start and finish line is ONE SECOND against the official time during the whole competition.

- 3.4	The maximum deviation of the transmitting periods is FIVE SECONDS against the official time during the whole competition. The maximum transmitting overlap of two transmitters is FIVE SECONDS.

## 4.	Other equipment

- 4.1	Any other equipment used by the organiser (service radio net, time-keeping system, computers, electronic marking devices etc.) shall not cause audible interference to competitors’ receivers.